VCON

Group Videoconferencing

Systems

Falcon IP™

User’s Guide

Modifications

The FCC requires the user to be notified that any changes or modifications made to

this system that are not expressly approved by VCON Ltd. may void the user’s

authority to operate the equipment.

Safety Notice

When you use the Falcon IP system, observe the following safety guidelines:

Make sure that the voltage of the power supply matches the AC power available at

your location:

_ 230V/50Hz in most of Europe, the Middle East and the Far East.

Make sure the monitor and attached accessories are electrically rated to operate

with the AC power available in your location.

Plug the accessories’ power cables into properly grounded power sources. These

cables are equipped with three-prong plugs to help ensure proper grounding.

Do not use adapter plugs or remove the grounding prong from a cable. If you must

use an extension cable, use a three-wire cable with properly grounded plugs.

iv VCON Falcon IP User’s Guide

Make sure that nothing rests on the cables and that the cables are not located where

they can be stepped on or tripped over.

Do not spill food or liquids on the system or accessories.

Keep the system away from radiators and heat sources, and do not block cooling

vents.

VCON Falcon IP User’s Guide 1-1

1 WELCOME TO GROUP VIDEOCONFERENCING

1-2 VCON Falcon IP User’s Guide

Remote Control

The hand-held remote control provides quick and easy access to all functions.

The most frequently-used functions are accessed by pressing the buttons located on

the top of the remote control. To avoid accidently pressing an inappropriate button,

less-frequently used buttons, such as the Menu button and the Soft Keys, are

shielded by a sliding cover.

Status View

Display Control

Arrow Buttons

Volume Control

Multipoint Control

Menu

Soft Keys

Preset Position Control

On-screen Help

Call Control

Dialog Control

Checkbox Button

Zoom Control

Mute Audio

Camera Select

Speed Dial

Still Image Control

Mute Video

con IP User’s Guide 2-1

2 GETTING STARTED

This chapter explains how to operate Falcon IP using its remote control.

2.1 Turning on the System

_ To start up Falcon IP

1. Make sure the system is set up, the TV monitor is on, and the cables are

connected properly. See “Setting Up the Basic Components” on

page 11-1.

2. On the back of the Main Unit, press the PWR switch on, marked (I).

Wait until the Guide Screen appears, containing the local video image.

_ The local video appears in the full-screen for 40 seconds.

_ Next, a new screen appears, advising you to wait.

_ Then the Guide Screen appears, and the system is ready to make calls.

Guide Screen

2-2 VCON Falcon IP User’s Guide

Chapter 2 Getting Started

2.2 Falcon IP Screens

The easy-to-use Falcon IP screens provide an efficient videoconferencing

environment. These are the elements displayed on Falcon IP screens:

Guide Screen

— Indicates that Falcon IP is operational and provides

a home screen for navigation.

— <Dial>, <Menu>, <Status> and <Help> are active.

— There are two sets of diagnostic icons on the Guide

Screen which indicate whether the lines are

connected (green) or unconnected (gray):

ISDN BRI lines (1 for Model 1; up to 3 for Model 3).

LAN (IP) connection.

Dialogs

Falcon IP’s dialogs provide on-screen information.

They can contain checkbox options, menu options, and

data-entry boxes. Press the arrow buttons to move

among the options and boxes.

Tabs

Some dialogs contain various tabs, which are related

sets of categorized options.

_ Press the up and down arrows to move among tabs.

_ Press the right arrow to open a dialog.

_ Press the left arrow to leave a dialog and move

among the tabs again.

Soft Key Guide

The four colored keys located on the covered part of the

remote control are the Soft Keys.

Soft Key functions change according to the status of the

system and the displayed dialog. The Soft Key Guide at

the top of the screen indicates the current functions.

Active Keys Guide

The Active Keys Guide on the left of the screen

indicates which buttons on the remote control are

currently active.

Chapter 2 Getting Started

VCON Falcon IP User’s Guide 2-3

2.3 Basic Functions

This section explains how to navigate in a screen, select options, and enter data.

_ To navigate among the options and boxes of dialogs and tabs

_ To select an option

_ To close a dialog

_ To enter numeric data

Press the arrow buttons.

1. Navigate to the menu or checkbox option.

2. Press the Checkbox button, located in the middle of the arrow

buttons.

To deselect an option, press the Checkbox button again.

To apply changes and then close the dialog, press <OK>.

-or-

To close a dialog without applying changes, press <Cancel>.

In any procedure in this User’s Guide when <OK> is mentioned,

<Cancel> is also possible.

1. Navigate to the data box in the dialog.

2. Press the number keys in the Number Pad.

If you make a mistake, press <Clear>.

2-4 VCON Falcon IP User’s Guide

Chapter 2 Getting Started

_ To enter alphabetical data

1. Navigate to the box in the dialog where the data is to be entered.

2. Press the yellow KEYBOARD Soft Key. The graphical keyboard

appears.

Graphical Keyboard for Alphanumeric Data Entry

To enter lower case letters, press the red LWR CASE Soft Key. To

enter upper case letters, press the red Soft Key again.

3. Press the arrow keys to navigate to the character you want to enter.

4. Press the Checkbox button to select a character.

If you make a mistake, select the DEL button on the graphical

keyboard.

5. Press <OK> to close the keyboard. The characters you selected are

entered in the box.

Chapter 2 Getting Started

VCON Falcon IP User’s Guide 2-5

VCON Falcon IP User’s Guide 3-1

3 CALL MANAGEMENT

This chapter explains how to answer, start, and end calls.

3.1 Answering Calls

You can set Falcon IP to answer all incoming calls automatically, or you can set it

to let you accept or reject each call.

Automatic Answer

If Automatic Answer is selected, a message appears when Falcon IP begins to

connect an incoming call.

Incoming Call is Automatically Answered

_ To set Auto Answer

Menu

Wait for the In Call icon to appear, indicating that the call has been

successfully connected.

1. Press <Menu>. The Menu opens.

Options Dialog - Models 1 and 3

3. If you want Falcon IP to answer all calls, select Auto Answer.

If you want to accept/reject calls as they come in, deselect Auto Answer.

4. Press <OK> to apply changes and close the dialog.

Accepting or Rejecting Incoming Calls

When Falcon IP is not set to Auto Answer, it rings to notify you of an incoming

call. A message, including the caller’s station name or address, asks if you want to

accept the call.

Accept Call Message

2. Select Options. The Options dialog opens.

To accept the call, press <OK>.

To reject the call, press <Cancel>.

Chapter 3 Call Management

VCON Falcon IP User’s Guide 3-3

3.2 Checking Call Details

When Falcon IP is engaged in a call, you can open the Status dialog and view the

details of the call, such as the identification of the remote caller (especially useful

when the call was automatically answered), the bandwidth used for the call, etc.

_ To open the Status dialog

This dialog varies, according to the type of call:

— ISDN (see below and page 3-4 to page 3-5),

— LAN (see page 3-6).

ISDN Status Dialog

The ISDN Status dialog has two tabs:

_ Lines - details of the local and remote parties’ lines engaged in the call.

_ Conversation - describes the status of the call’s audio and video.

NOTE

The Status dialog appears differently if you are not in a call. See

“Testing Network Configuration” on page 8-9, for Status details when

not in a call.

Press <Status>. The Status dialog opens.

3-4 VCON Falcon IP User’s Guide

Chapter 3 Call Management

Lines Tab

Status Dialog - Lines, During a Call, Model 3

Conversation Tab

Status Dialog - Conversation, During a Call, Model 3

(Number) The ISDN numbers of the remote party.

Synchronized The line is engaged in the current call.

Not Synchronized

The line is not engaged in the current call. It may be

disconnected or experiencing problems.

Dialing The system is attempting to connect over this line.

Chapter 3 Call Management

VCON Falcon IP User’s Guide 3-5

Remote ID The Station Name or first ISDN number of the remote

party.

Conv Protocol H.320. Indicates that the remote party also supports the

ITU-T Recommendation for standard ISDN calls.

Bandwidth Model 1 - bandwidth is 128, if both ISDN lines

successfully connected.

Model 3 - bandwidth is 384, if all six ISDN lines

successfully connected.

Any other value indicates that one or more of the ISDN

lines failed to connect. The bandwidth is 64 multiplied by

the number of connected lines.

Audio Mode G.728 is used for low bandwidth calls (128 - 320 Kbps).

G.722 is used for high bandwidth calls (384 Kbps).

G.711 is used if the systems engaged in the call do not

have any other common audio support.

Audio Rate The bandwidth (Kbps) needed to transmit audio; either 16

Kbps (for G.728) or 56 Kbps (for G.722 or G.711).

Video Mode The mode used for compressing video:

H.263 - preferred mode.

H.261 - used if the remote system can’t support H.263.

3-6 VCON Falcon IP User’s Guide

Chapter 3 Call Management

LAN Status Dialog

The LAN Status dialog is available when Falcon IP is in a LAN call. The Audio

Mode, Audio Rate, and Video Mode parameters are described on page 3-5.

Status Dialog During a LAN Call

Video Rate The dynamic bandwidth currently being used for video.

Because LAN calls do not have a static bandwidth

(unlike ISDN calls), this constantly changing property is

a valuable indicator of the network resources available

for the call.

Gatekeeper Indicates if the system is registered or not registered

(see page 8-4 for Gatekeeper details).

Remote IP

Address

The IP Address of the remote party.

Remote Name The Station Name of the remote party, if available.

Chapter 3 Call Management

VCON Falcon IP User’s Guide 3-7

3.3 Starting Calls

To start a videoconference, dial an appropriate address (see the table below) in one

of the following dialogs:

_ Manual Dial - see “Manual Dialing” on page 3-8.

_ Phone Book - see “Phone Book Dialing” on page 3-12.

_ Speed Dial - see “Speed Dialing” on page 3-13.

Dialing Options - ISDN Dialing Types

Falcon IP supports various methods of connecting ISDN calls. These dialing

methods are available to any Falcon IP unit with ISDN BRI lines.

NOTE

Before calling, you may choose to mute the microphone (see “Adjusting

the Volume” on page 7-6) to ensure that the remote party does not

overhear a local conversation when the connection is first made.

Dialing Type Explanation Advantages

By Channel

(each ISDN number indicates a channel)

Enter the destination ISDN numbers and then dial. Each ISDN number connects and

synchronizes individually to the destination.

Call starts when first line is connected.

VCON products support this dialing option for all ISDN calls up to 384Kbps (6 channels).

Bonding Enter only the first ISDN number and then dial. All other channels then connect and synchronize

together.

Only one ISDN number needs to be dialed when Bonding is selected.

Supported by all vendors.

Instant All

(Menu/ Options)

Works with both By Channel and Bonding. ISDN numbers connect as a unit and then synchronize.

Faster initial connection. Better connection for international calls.

Restricted

The bandwidth of the call is based upon 56 Kbps, rather than the default 64.

Must be used to reach a destination that is connected over a Restricted network.

3-8 VCON Falcon IP User’s Guide

Chapter 3 Call Management

Dialing Options - LAN Address Types

The following table defines the network address types supported by Falcon IP.

Enter one of these address types in the LAN Manual Dial dialog or in a LAN -

New Entry dialog of the Phone Book.

Manual Dialing

Dial from the Manual Dial to quickly call a contact. Use the Manual Dial as you do

a telephone: enter the number or address and press <Dial>. Falcon IP dials the

destination, but does not store the number.

For frequently repeated calls to the same destination, it is more efficient to store

entries in the Phone Book (see “Phone Book Dialing” on page 3-12).

_ To dial in the Manual Dial dialog

Address

Type

Example Applicable When

IP

Address

172.16.1.33 Always applicable. Usually dynamic, changing

without notification, so the other address types

are often preferred.

DNS

Name

WERHEE.gov.local The IP addresses of a DNS server and of a WINS

server are configured. The Falcon IP Station

Name is the first part of the DNS address.

Station Name

WERHEE Both Falcon IP and the remote system are

registered with the Gatekeeper. Any string of

characters beginning with an alphabetical letter.

User Number

(E.164)

2001 Both Falcon IP and the remote system are

registered with the Gatekeeper. Any number.

VCON Falcon IP User’s Guide 3-9

Manual Dialing ISDN Numbers

Press any number to open the ISDN tab of the Manual Dial. This dialog varies between the models:

_ If you have a Model 3, you can dial up to six channels.

Manual Dial - ISDN, Model 3

_ To dial ISDN numbers

1. Press the right arrow button and enter the first ISDN number.

2. Press the down arrow button and enter the next number. (see the next page for instructions on copying numbers).

If both channels have the same number, copy the first number into the box of the second number.

3. Continue with steps 1 and 2 for all the numbers. If the remote party is connected to a Restricted network (56 Kbps per channel, instead of 64 Kbps), select Restricted.

4. Press <Dial>. Falcon IP dials the destination.

3-10 VCON Falcon IP User’s Guide

_ To dial with Bonding

1. Press the down arrow button and enter the ISDN number.

2. Press the right arrow button to highlight Bonding.

3. Press the checkbox button to select Bonding. The Manual Dialing

Bonding options appear in the dialog.

_ To choose a bandwidth

1. Press the down arrow button. Bandwidth is highlighted. The available maximum is selected by default.

2. Press the right arrow button. The Bandwidth value changes.

3. When you see the bandwidth that you want, press <Dial>.

4. Press <Dial>. Falcon IP dials the destination.

NOTES

If you select Restricted, the possible bandwidth values are different,

based on 56 Kbps per channel, rather than the default 64.

If you want to return to the default ISDN Manual Dial dialog, deselect

Bonding in the Manual Bonding dialog.

Chapter 3 Call Management

VCON Falcon IP User’s Guide 3-11

Manual Dialing LAN Addresses

Open the LAN tab of the Manual Dial.

Manual Dial - LAN

_ To dial LAN addresses

1. Enter the LAN-based address.

To set a maximum bandwidth for this call, highlight Bandwidth and press the right arrow button until the  value you want appears.

2. Press <Dial>. Falcon IP dials the destination.

3-12 VCON Falcon IP User’s Guide

Chapter 3 Call Management

Phone Book Dialing

Phone Book Entries in Alphabetical Tabs

_ Maintain Phone Book entries for frequently-dialed addresses to make dialing

more efficient. You can dial from the Phone Book after you create an entry

(see “Adding New Phone Book Entries” on page 4-2).

_ To dial using the Phone Book

1. Press <Dial>. The Phone Book opens.

2. Open the tab that contains the entry you want to dial (see “Falcon

IP Screens” on page 2-2 for tab-navigation instructions).

3. Highlight the entry. The calling details of the entry appear under

the Phone Book: ISDN number or LAN address, and bandwidth.

4. Press <Dial>. Falcon IP dials the destination.

NOTE

If you press <OK> instead of <Dial>, the Phone Book closes without

dialing the number.

Chapter 3 Call Management

VCON Falcon IP User’s Guide 3-13

Speed Dialing

Speed Dial

In the Speed Dial, enter the numbers or addresses of your most frequently dialed destinations (up to nine). You can dial from the Speed Dial by pressing a single button, and you don’t have to search tabs for an entry.

_ To dial a number from the Speed Dial

_ To add an entry to the Speed Dial list

_ To add a new Phone Book entry to the Speed Dial list, select Speed Dial in the New Entry dialog .

_ To add an entry after creating it, open the Edit Entry dialog and select

Speed Dial in the Edit Entry dialog .

The Speed Dial list can hold up to nine entries, plus the zero entry for redialing the last dialed number.

1. When the screen is clear of dialogs, press <#>. The Speed Dial appears.

2. Press a Speed Dial entry number (1 to 9). Falcon IP dials the destination.

If you need to scroll down the list, press the down arrow button until the entry you want is highlighted. Press <OK> to dial.

3-14 VCON Falcon IP User’s Guide

_ To delete a Speed Dial entry

1. Highlight the entry you want to delete and then press the red DELETE Soft Key. A message asks for confirmation.

2. Press <OK>. The entry is deleted from the Speed Dial list (the Phone Book entry is not affected).

Redialing

Press one of the following buttons to redial the last number dialed:

3.4 Ending Calls

_ To end a call

From the Speed Dial, press <0> (zero).

From the Phone Book, press the yellow REDIAL Soft Key.

From the Manual Dial, press the red REDIAL Soft Key.

1. Press <Hang up>. A message asks for confirmation.

2. Press <Hang up> again or <OK>. The call is terminated.

NOTE

To cancel the Hang up command and continue the videoconference,

press <Cancel> in response to the confirmation message. Guide 4-1

4 MANAGING THE PHONE BOOK

Store frequently-called numbers as entries in the Phone Book. This easy procedure makes videoconferencing with your favorite contacts easy and convenient.

_ To open the Phone Book

The Phone Book

Press <Dial>. The Phone Book opens.

4-2ser’s Guide

Chapter 4 he Phone Book

4.1 Adding New Phone Book Entries

Before you can dial from the Phone Book, you must create Phone Book Entries.

_ To create a new Phone Book entry

_ To add an entry with ISDN numbers, see below.

_ To add an entry with an IP address, see “Adding LAN Entries” on page 4-4.

Adding ISDN Entries

New Entry - ISDN, Model 3

Press the red NEW Soft Key. The Phone Book New Entry dialog opens to the previously-used tab (either ISDN or LAN).

1. Open the ISDN tab.

2. Enter a name for this entry . The name must begin with a letter; it cannot begin with a number or symbol.

Chapter 4 Managing the Phone Book

VCON User’s Guide 4-3

3. Press the down arrow button and enter the first ISDN number. Press the down arrow button and enter the next number(s).

— If a BRI has the same ISDN number for both channels, press the green

COPY Soft Key to copy the previously entered number into the current

box.

— If the remote party is connected to a Restricted network, select Restricted.

— To add this entry to Speed Dial list, select Speed Dial.

— To add the Bonding option to this entry, select Bonding.

The New Entry Bonding dialog opens.

New Entry - ISDN with Bonding

Make sure that the ISDN number represents the first channel.

If you want to select a Bandwidth other than the available maximum,

press the down arrow button and then press the right arrow button

until you see the bandwidth that you want.

4. Press <OK> to add the entry and close the dialog.

4-4 VCON ’s Guide

Chapter 4 Managing the Phone Book

Adding LAN Entries

New Entry -LAN

If the contact has an IP Address or LAN-based name or number, add a LAN entry.

3. Enter an address for this contact.

— To set a specific bandwidth, press the down arrow key to highlight

Bandwidth and then press the right arrow key until the maximum

bandwidth you want to use for calls to this contact appears.

— To add this entry to Speed Dial list, press the down arrow key to highlight

Speed Dial and press the Checkbox button.

4. Press <OK> to add the entry and close the dialog.

1. If the LAN tab is not open, press the down arrow button to highlight

LAN on the tab labels, and then press the right arrow button. The

LAN tab opens.

2. Enter a name for this entry. The name must begin with a letter; it cannot begin with a number or symbol.

Chapter 4 Managing the Phone Book

VCON User’s Guide 4-5

4.2 Editing and Deleting Phone Book Entries

You can edit or delete entries at any time.

_ To edit an entry

_ To delete an entry

1. Press <Dial>. The Phone Book opens.

2. Highlight the entry you want to edit and then press the green

EDIT Soft Key. The Phone Book Entry dialog opens.

3. Edit the data in the dialog

To erase the character to the left of the cursor, press <Clear>.

4. Press <OK> to apply the changes and close the dialog.

1. From a Phone Book tab, highlight the entry you want to delete

and then press the blue DELETE Soft Key. A message asks for

confirmation.

2. Press <OK> to delete the entry and close the dialog.

VCON Falcon IP User’s Guide 5-1

5 MULTI-PARTY CALLS WITH MCUS

Falcon IP supports multi-party calls over Multipoint Control Units (MCU).

In a point-to-point call, one videoconferencing endpoint calls another.

In an MCU call, multiple videoconferencing endpoints engage in separate calls

with an MCU, which then enables all parties to be seen and heard by all the others.

5.1 Multipoint Control Units (MCUs)

An MCU is a network device used to connect multiple endpoints in call. Endpoints

either call the MCU or they accept a call from the MCU. Then the MCU engages in

a separate call with each system simultaneously. When one side transmits, the

MCU receives the multimedia stream, multiplexes the audio and video, and then

sends the resulting streams to all the endpoints in the session.

5-2 VCON Falcon IP User’s Guide

Chapter 5 Multi-Party Calls with MC

MCU Management

The MCU may manage the session in different ways:

H.323 and H.320 MCUs

Falcon IP can communicate with MCUs operating according to the standards set by

the H.323 and H.320 Recommendations.

_ If you engage in a multipoint call over the LAN (contact an MCU’s IP

address), the MCU managing the videoconference is an H.323 MCU.

_ If you engage in a multipoint call over ISDN lines (contact an MCU’s ISDN

numbers), the MCU managing the videoconference is an H.320 MCU.

5.2 Participating in a Multipoint Call

Before you call an MCU, consult with the system administrator.

_ To participate in a multi-party videoconference

1. Press any number pad button when the TV monitor is clear of dialogs. The Manual Dial opens.

2. Enter the MCU’s address in the box.

3. Press <Dial>. Falcon IP calls the MCU. The MCU receives your video and audio and sends it to all the connected systems.

Voice Activation

The MCU transmits to all endpoints the

audio and video of the party currently

speaking.

Continuous Presence

The MCU continuously receives and

transmits from multiple endpoints.

NOTE

Various MCU brands have different session management designs. The

display you receive from your MCU may differ from those shown here.

Chapter 5 Multi-Party Calls with M

VCON Falcon IP User’s Guide 5-3

If the MCU is an H.320 MCU, it may be able to turn over the management of the

session to a participant, who then becomes the Chair. Additionally, participants

have some control over what they see.

5.3 Managing an H.320 Multipoint Call

The most important feature of managing an MCU session is controlling which of

the multiple participants is seen and heard.

Chapter 5 Multi-Party Calls with

VCON Falcon IP User’s

6.1 Calling Through Gateways

However, at some point, you may be unable to take advantage of one of the communication modes (IP or ISDN). If this happens, you can still meet with your contacts by calling through an H.323/H.320 (IP/ISDN) Gateway.

_ If your LAN connection is down, you can call the Gateway through your ISDN

lines and reach H.323 systems (IP addresses and other LAN-based addresses).

_ If your ISDN lines are down, you can call the Gateway through your LAN

connection and reach H.320 systems (ISDN numbers).

Falcon IP to Gateway to ISDN - Falcon IP to Gateway to IP

6-2 VCON Falcon IP User’s Guide

Chapter 6 Calling Other Networks

_ To call through a Gateway

1. Open the Manual Dial dialog and enter the number as directed by the system administrator.

2. Press <Dial>.

The Gatekeeper, a network services application installed on the Gateway, receives the call. It recognizes the syntax that the system administrator instructed you to enter, and it passes the call onto the Gateway. You must be registered with a Gatekeeper to reach a Gateway.

The Gateway calls the destination, and when it receives an answer, it

begins the videoconference, continuously translating between H.320

(ISDN) and H.323 (IP).

7 VIDEO AND AUDIO ADJUSTMENTS

Falcon IP offers basic and powerful options that enable you to control and

customize the video and audio. Some of these adjustments should be made before

entering into a videoconference, others are available only during a call.

7.1 Before a Call

You can control and adjust various camera, display, and audio settings before

entering a videoconference.

Adjusting the Video Settings

You can adjust the brightness, color, contrast and hue of the local video, either

prior to or during a videoconference. To save time during a videoconference, adjust

these properties before you begin a call. You can adjust them again at any time.

_ To adjust video settings

Menu

1. Press <Menu>. The Menu opens.

2. Select Camera. The Brightness dialog opens.

Chapter 7 Video and Audio Adjustments

Brightness Dialog

3. Press the up and down arrow keys to switch between Brightness, Color, Contrast, and Hue.

4. To increase the setting, press the right arrow key.

To decrease the setting, press the left arrow key.

To return the setting to its default value, press the red DEFAULT Soft Key.

5. Press <OK> to apply the changes and close the dialog.

VCON Falcon IP User’s Guide 7-3

Chapter 7 Video and Audio Adjustments

Setting Video Options

Before a videoconference begins, you may enable advanced video options.

_ To set video options

1. Press <Menu>. The Menu opens.

2. Select Video. The Set Video dialog opens.

Set Video Dialog

3. Press <OK> to apply the changes and close the Set Video dialog.

Dual Monitor

In a dual monitor configuration, you connect two TV monitors rather than just one TV.

One TV monitor is used for remote video, and the other is used for local video.

Select this option only if you have two TV monitors connected. If you enable Dual Monitor for a single

Composite monitor, the Falcon IP dialogs will not appear.

Camera Control By Far End

Select this option if you want to give the remote party the following controls:

Control local main camera during a call

Select an optional local video source (CAM1, CAM2,

VCR)

Generate a still image from the local video source.

Chapter 7 Video and Audio Adjustments

7-4 VCON Falcon IP User’s Guide

Controlling the Camera

You can position the Falcon IP’s internal camera at any time, either prior to or

during a call. You can pan, tilt, and zoom the camera every time you want to

change the local display, or you can save preset positions.

Manual Positioning

When the screen is clear of dialogs, the arrow buttons control the camera.

_ To manually position the camera

Preset Positioning

You can save ten different camera preset positions, and then restore the camera to these positions as needed.

For example, if there are five people in the conference room, you can set a preset position for each person. Then you can recall the preset positions as needed to quickly focus on the person currently speaking.

_ To preset a position

_ To restore a camera to a preset position

To pan the camera left and right, press the left and right arrows.

To tilt the camera up and down, press the up and down arrow.

To zoom the camera in and out, press the Zoom <+> and <-> buttons.

1. Manually position the camera.

2. Press Presets <Set>.

3. Press a number in the number pad. Each of the numbers between zero and nine can represent a preset position.

1. Press Presets <Recall>.

2. Press the number in the number pad corresponding to the preset position. The camera pans, tilts and zooms to the preset position.

VCON Falcon IP User’s Guide 7-5

Chapter 7 Video and Audio Adjustments

Setting Audio Options

Prior to and during a videoconference, you may enable advanced audio options.

_ To set audio options

1. Press <Menu>. The Menu opens.

2. Select Audio. The Set Audio screen opens.

Set Audio

3. Choose an audio input device.

4. If there is a VCR connected to the Falcon IP, choose a mixing option.

Tabletop mic To use the supplied table top microphone or other audio device connected to the Falcon IP’s MIC connector.

Line level To use an optional audio input device (such as a mixer) connected to the Falcon IP’s VCR connectors.

No mixing Both parties hear each other’s audio only.

VCR Record

Both parties hear each other’s audio while a VCR records both of them.

VCR Playback

Both parties hear each other’s audio and the playback of the VCR cassette Guide

5. Set audio options for special room environments.

6. Press <OK> to apply the changes and close the dialog.

7.2 During a Call

Some video and audio options are available only during a call. These features are easy to use; you will probably use most of them during any typical call.

Adjusting the Volume

You can adjust the volume of the incoming audio, and you can mute both the speakers and the microphone.

_ To adjust the volume of the local speakers

Set Volume Dialog

AEC Automatic Echo Cancellation cancels the echo created when the microphone picks up audio from the speakers. Deselect AEC if an external AEC device is in use.

AGC Automatic Gain Control amplifies audio pickup to improve audio in big rooms.

ANS Automatic Noise Suppression blocks noise.

1. Press <Volume>. The Set Volume dialog appears.

2. To increase the volume, press Volume <+>.

To decrease the volume, press Volume <->.

3. Press <OK> to apply the change and close the dialog.

VCON Falcon IP User’s Guide 7-7

_ To mute the speakers

_ To mute the microphone

Selecting a Display Mode

Different display modes are available during a call.

_ Local - your party

_ Remote - the other party

_ PIP - Picture In Picture. A small inset of one party appears over the full-screen display of the other party.

When a call starts, PIP mode is activated: the remote party is displayed on the full screen and the local party is in the PIP.

You can switch the displays in PIP mode: you can view the remote party in the PIP and your party on the full-screen.

Additionally, you can turn PIP mode off to see a full-screen display of either party.

Press Mute <Speaker>. An animated picture on the bottom of the screen indicates that the speakers are muted.

To restore the speakers, press Mute <Speaker> again.

Press Mute <Microphone>. An animated picture on the bottom of the screen indicates that the microphone is muted.

To restore the microphone, press Mute <Microphone> again.

To switch between local and remote displays, both in PIP mode and in full-screen mode, press <Display>.

To turn PIP mode on or off, press <PIP>.

Chapter 7 Video and Audio Adjustments

7-8 VCON Falcon IP User’s Guide

Moving the PIP

You can move the PIP to any location on the screen. For example, you would move

the PIP if someone in the remote party stood up and the PIP covered his/her face.

_ To move the PIP

Adjusting Quality of LAN Calls

Falcon IP can automatically adjust its video and audio for dynamic IP bandwidth.

However, you may want to adjust the quality-control mechanisms yourself if you

experience the following problems:

Parts of the video are covered with blocks (low quantization control).

The audio and video fall out of synchronization (low quality lip synch).

The video freezes and then doubles up frames (inefficient buffering).

_ To adjust videoconference quality for LAN calls

1. Press <Menu>. The Menu opens.

2. Press the red MOVE PIP Soft Key. A direction guide appears.

3. Press the arrow buttons to move the PIP around the screen.

4. When it is positioned where you want it, press <OK>.

1. In the H.323 tab of the Set Network dialog, select Enable Lip

Synchronization Mechanism and deselect Automatic Buffering

Control (this enables adjustable Buffering Control.

2. Press <Status>. The Status dialog opens.

Status Dialog - LAN

Lip Synch Slider of the Adjustment Dialogs

During a call, the red ADJUST Soft Key is available. You can access the Adjustment sliders only during a call.

3. Press the red ADJUST Soft Key. The first slider appears.

4. Press the up and down arrow keys to view and adjust the different sliders:

— For best results, start the sliders at one end and gradually adjust them.

— To reset the default values, press the red DEFAULT Soft Key.

5. Press <OK> to apply the changes and close the dialog.

Selecting a Camera

The camera select option is always available. At any time, you can select different local video sources. During a call, you can change both local and remote video sources.

_ To select a camera

Local Camera Select Dialog

Quantization Press the left arrow key to clear up blocks in the video.

Press the right arrow key for better motion.

Lip Synch Press the left arrow key if sound comes before motion.

Press the right arrow key if motion comes before sound.

Buffering Press the left arrow key for a more realistic video.

Press the right arrow key to clear up video freezing and jumping.

1. Press <CAM>. The Camera Select dialog opens. The currently selected camera is highlighted.

2. Press the left or right arrow buttons to switch between the local optional video sources (up to four):

— Falcon IP main camera

— A video source connected to the VCR connector.

— A Composite camera connected to the CAM1 connector.

— An S-Video camera connected to the CAM2 connector.

3. Press the down arrow key to view the remote camera options.

Remote Camera Select Dialog

4. Press the left or right arrow buttons to switch between the remote cameras.

5. Press <OK> to select the highlighted camera and close the dialog.

Muting the Video

Mute the local outgoing video to transmit a frozen image in place of the local

video. The view you see of the remote party remains unaffected as the conference

continues, and both incoming and outgoing audio continue as usual.

_ To mute the outgoing video

Press <Mute Video>. An animated icon on your screen indicates that

local video is being muted.

To restore the outgoing video, press <Mute Video> again.

Chapter 7 Video and Audio Adjustments

8 NETWORK CONFIGURATION

This chapter explains how to set up Falcon IP to operate with your network

connections. Network options may be edited at any time, either from the Set

Network dialog described in this chapter or from a remote station .

8.1 General Network Configuration

_ To set up network connections

1. Press <Menu>. The Menu appears.

2. Select Network. The Set Network dialog appears with the

Location tab open.

— Press the up and down arrow buttons to move among the tabs.

— Press the right arrow button to open a tab.

— Press the left arrow to leave a tab.

3. After setting network properties in the tabs, press <OK>.

A message asks you to wait while the system restarts itself.

8-2 VCON Falcon IP User’s Guide

8.2 Understanding Network Properties

This section describes the required network information.

Location Tab

Set Network Dialog - Location

Station Name The Station Name is the identification of the Falcon IP.

Consult with the system administrator before modifying

this box.

If the system is registered with the WINS server or a Gatekeeper, Falcon IP can use station names instead of IP addresses for starting and receiving calls. When you register with a Gatekeeper, the station name is entered as your User Name.

If the system has access to a Domain Name Service (DNS) server, the Station Name is also the DNS Host

Name of the system. Enter the Host Name, then a dot, then the Domain Name:

StationName.Company.com

If you are in a Multipoint videoconference with an MCU, the Station Name identifies the Falcon IP in the

Participants List of the MCU dialog.

VCON Falcon IP User’s Guide 8-3

Chapter 8 Network Configuration

LAN Tab

Set Network Dialog - LAN Tab

In the LAN tab, enter the system’s addressing information.

Obtain IP Auto The IP addresses are automatically entered after restarting

Falcon IP. This feature is successful only if the system has access to a DHCP server.

If you deselect this checkbox, the remaining boxes are available.

IP Address Enter a permanent IP Address here. If Falcon IP receives an address from the DHCP server, it is a temporary address. If you manually enter an address in this box, it is a permanent address.

Subnet Mask Enter your company’s subnet mask.

DNS Server & WINS Server

Enter the IP Addresses of the DNS server and of the WINS server. Registering with these servers enables Falcon IP to translate names to IP addresses.

Domain

Enter the DNS domain name of your company

(yourcompany.com).

Default Gateway

Enter the IP address of the network Gateway router to be able to

send and receive calls between subnets.

Chapter 8 Network Configuration

8-4 VCON Falcon IP User’s Guide

H.323 Tab

H.323 is the standard for videoconferencing over IP networks. An important component of IP networks is the Gatekeeper. It is a network management mechanism for video, audio, and data streaming over LANs and WANs.

When Falcon IP registers with a Gatekeeper, it receives the following services:

Translation of addresses. Every registered endpoint can reach every other with User Numbers or User Names because the Gatekeeper translates them into IP

Addresses. The User Name of Falcon IP is the Station Name.

Access control. The Gatekeeper secures communication within its zone of registered endpoints by rejecting calls from outside the zone.

Bandwidth control. The Gatekeeper stops calls that would have congested the network.

Zone management. The Gatekeeper manages all endpoint communication, including Gateways as well as videoconferencing stations.

NOTE

If you want to register with a protocol-translation gateway/gatekeeper

(such as a Radvision Gateway) to be able to send and receive calls between

networks (IP and ISDN), enter the IP Address of the gateway in the

Gatekeeper Configuration of the H.323 tab.

VCON Falcon IP User’s Guide 8-5

Chapter 8 Network Configuration

Gatekeeper Configuration

Check this box to register with a Gatekeeper.

IP Address Enter the IP address of the Gatekeeper.

User Number Change the default example to a numeric string that will identify this system with the Gatekeeper and other contacts. Make sure that this number is unique in the Gatekeeper’s database and that it is different from the Station Name.

Enable Lip Synchronization Mechanism

Select this option to synchronize the audio and video of a LAN conversation. You can adjust the lip synch during a call.

Automatic Buffering Control

Buffer Control adjusts the video streams for the available dynamic bandwidth of IP networks.

Select this option to make the buffer control automatic.

Deselect it to make it adjustable during LAN conversations.

Chapter 8 Network Configuration

8-work Configuration

8.3 Testing Network Configuration

After you configure the Set Network dialogs and restart Falcon IP, you can check

that the changes that you made were applied.

_ To check network configuration

_ To check the status of the ISDN lines, see “ISDN Status_ To check the network connection, see “LAN Status”

ISDN Status

Status Dialog - ISDN, When Not in a Call

Press <Status>. The Status dialog appears. It has ISDN and LAN tabs.

Checking Falcon IP is checking the line. Wait until the status changes.

Connected The line is connected properly.

Disconnected The line is not connected properly.

Chapter 8 Network Configuration

8-10 VCON Falcon IP User’s Guide

LAN Status

Status Dialog - LAN, When Not in a Call

_ Check that the IP Address displayed is the one that you set or received for the system.

_ Check that the system is Registered or Not Registered with the Gatekeeper, depending on the H.323 options that you chose.

If, in the H.323 tab, you selected Gatekeeper Configuration and entered the correct IP address, you should be registered. If you are not, change the User Number that you entered. You might have entered one that is already taken.

For more help, press the green DIAG button to open the Diagnostics dialog.

VCON Falcon IP User’s Guide 9-1

Chapter 9 Web-Based Remote Management

9-6 VCON Falcon IP User’s Guide

11 SETTING UP FALCON IP

This chapter explains how to set up the Falcon IP system and connect accessories.

Before setting up Falcon IP, make sure you have the necessary components--follow

the checklist provided in the package. If anything is damaged or missing, contact

your local VCON distributor.

11.1 Setting Up the Basic Components

Carefully unpack the Main Unit, and position it on a stable horizontal area.

Single and Dual TV Monitor Configurations

VCON supplies two types of cables to connect between the Main Unit and the

VIDEO-IN connectors on the TV monitor.

_ To connect a single TV monitor

1. Use either the S-Video (CAB72009) or the Composite cable (CAB72008),

depending on the TV monitor that you have. If your TV offers both types

of connections, use the S-Video type for higher quality.

2. Connect the red and white audio cables joined with the video cable that

you decide to use.

_ To connect dual TV monitors

The Dual Monitor configuration displays the remote video on the S-Video TV, and

the local video on the Composite TV.

1. Connect the Composite cable (CAB72008) between the Main Unit and the

TV monitor that will show the local video.

2. Connect the S-Video cable (CAB72009) between the Main Unit and the

TV monitor that will show the remote video and the GUI.

3. Connect the audio cables between the Main Unit and the TV monitor that

will show the remote video.

Basic Falcon IP Connections

The illustration on the next pages shows the basic accessories connected to the

Main Unit.Chapte11 Setting p Falcon IP

11-2 VCON Falcon IP User’s e

CAB72009 or CAB72008 CAB42001

ISDN CABLES -1, 2, or 3 to outlet

Chapter 11er’s Guide

Optional Falcon IP Connections

VCR

CABLES

CAB72010 CAB90051

VCON Falcon IP User’s Guide 11-5

_ To connect a VCR to the Main Unit

1. Obtain the cables supplied with the VCR.

2. Connect the top three cables for Playback.

3. Connect the bottom three cables for Record.

_ To connect an optional camera

_ Connect the appropriate cable, either CAB72010 or CAB90051, according to the capabilities of the camera.

11.2 Turning on Falcon IP

1. Press the PWR switch on (I).

2. Verify that the green LED is lit, indicating that the system is ready.

3. Turn on the TV monitor(s) and any connected optional accessories. Wait for the Guide Screen to appear.

VCON Falcon IP User’s Guide 12-1

12 TROUBLESHOOTING

This chapter provides simple solutions for common oversights and issues.

If you are unable to find a solution to your problem, call your local VCON Distributor. The solutions are grouped by the type of equipment affected:

_ Main Unit and Accessories,

_ IP Network,

_ ISDN Lines.

12.1 Diagnostics Dialog

Before you begin searching for the source of a problem, you can check the

Diagnostics dialog. It describes any network and server problems that may exist.

_ To open the Diagnostics Dialog

Status Dialog

1. Press <Status>. The Status dialog opens.

Chapter 12 Troubleshooting

12-2 VCON Falcon IP User’s Guide

Diagnostics Dialog

If there is a problem described in the Diagnostics dialog, consult with the system administrator to find a solution.

If the problem you are trying to solve is not described in the Diagnostics dialog, try to find the solution in the following sections.

2. Press the green DIAG Soft Key. The Diagnostics dialog opens.

You can restore the factory-set defaults for the system, by pressing the blue SET DFLT Soft Key.

A confirmation message appears. Click OK if you are sure you want to cancel all changes made to the Falcon IP options and settings and restore the original defaults.

Falcon IP restarts.

If you have a problem with the Phone Book, you can erase it completely, by pressing the green ERASE PBK Soft Key.

A confirmation message appears. Click OK if you are sure you want to erase all Phone Book entries.

VCON Falcon IP User’s Guide 12-3

Chapter 12 Troubleshooting

12.2 Main Unit and Standard Accessories

Blank Screen Solution

No video appears on the TV screen after more than one minute.

1. Verify that the TV monitor is on.

2. Verify that all cable connections are correct and secure.

3. If the selected video source is an optional camera, verify that it is turned on and operating properly.

No Menus Solution

Guide Screen or Menu and Phone Book do not appear.

1. Unplug the Composite VIDEO-IN connector and plug it into the VCR VIDEO-OUT connector.

If the Menus now appear, open the Set Video dialog and deselect Dual Monitor.

2. Verify that the batteries are inserted properly in the remote control.

No Audio Solution

While in a call, you can’t hear the remote party, and/or they can’t hear you.

1. Verify that the volume on the local TV set is set to an audible level.

2. Press <Volume> and make sure that the volume level of Falcon IP is audible.

3. Verify that the microphones and TV sets on both the local and remote sides are connected properly

and securely.

4. Press <Menu> and select Audio. Make sure that the correct audio input device and mixing mode are selected.

No Video Solution

You cannot see video from an optional camera or a VCR.

1. Verify that the cables from the video source are connected correctly between the main unit and the video source.

2. Press <CAM> and make sure that the appropriate video source is selected.

Chapter 12 Troubleshooting

12-4 VCON Falcon IP User’s Guide

12.3 IP Network

Before attempting to troubleshoot, and after checking the Status and the Diagnostics dialogs, verify that:

_ The LAN cable is securely connected between the Main Unit and the network outlet.

_ The LAN connection is operational, as shown by the green L icon at the top of the Guide Screen. .

No Still Image Solution

The remote party is unable to request a still image.

_ Open the Set Video dialog (select Video from the Menu) and select Camera Control By Far End.

Cannot Connect Solution

You dial over IP and it does not connect.

1. Verify with the system administrator that the Falcon IP has an IP Address and is registered

with the LAN segment.

2. Verify that the remote party is registered with the same or a neighboring Gatekeeper.

Cannot Take Gatekeeper Calls Solution

You can receive IP calls, but you cannot receive calls that go through the Gatekeeper with your

User Name or User Number  (E.164 number).

1. Press <Status>. If the LAN tab of the Status dialog says Gatekeeper: Not Registered, you cannot communicate with registered systems.

2. Verify that you entered a User Number (E.164 number) in the H.323 tab, and check that the User Name (Station Name) in the Location tab of the Set Network dialog is legal.

3. Verify that you gave the correct User Number and User Name (local Station Name) to the remote party.

VCON Falcon IP User’s Guide 12-5

Chapter 12 Troubleshooting

12.4 ISDN Network

Before attempting to troubleshoot, and after checking the Status and the Diagnostics dialogs, verify that:

_ The ISDN lines are securely connected between the Main Unit and the outlets.

_ The ISDN lines are operational, as indicated by the green 1, 2, and 3 icons at the top of the Guide Screen.

_ The  lines were configured correctly in the Set Network dialog .

Cannot Call Some Sites Solution

You can call most sites, but not others.

1. Have the remote side call you. If the call is successfully made, go to the next step. If this is unsuccessful, go to Step 5.

2. Select Restricted in the New Entry dialog or the Manual Dial dialog. Call this site again. If unsuccessful, go to the next step.

3. If the remote system is a VCON system, then call without Bonding. If this is unsuccessful, call using Instant All.

4. If the remote system is not a VCON system, verify that they can receive a call using the H.320 standard.

5. Run a loop back test: dial your 2nd, 4th, and 6th ISDN numbers. If Falcon IP successfully calls itself, go to the next step.

6. Ask the remote party to run a loop back test, to check the operation status of their lines.

Disconnected Status Solution

You know that the lines are connected, but the Status dialog displays “Disconnected”.

1. If you are using network termination equipment, verify that it is turned on.

2. Verify that you chose the correct Switch type in the Set Network dialog.

VCON Falcon IP User’s Guide I