1. Sony Mixer

2. VCON Camera & Voice Finder

3. Falcon Installation Document

1.  Remove the Falcon unit from the box.

2.  Install equipment into the cabinet as per the Cabinet layout Diagram.

3.  Place Falcon on the shelf of the cabinet.

4.  Place the batteries into the Remote Control.

5.  Install the Sony Mixer into the Cabinet as per the Cabinet Layout Document.

6.  Install the Plasma’s Receiver into the Cabinet as per the Cabinet Layout

Diagram.

7.  Mount the Plasma Screen as per the Room Layout Diagram.

8.Connect the system by using the Falcon Set Up Document.

Falcon Set up Document

| Connecting Cable Type/Name | From which Connection | To which Connection |
| --- | --- | --- |
| RJ45 Lan Cable | Falcon Lan Connection | Lan Connector on the Network |
| Tripple RCA Cable Yellow, Red & White | Falcon TV- White Audio Cable | ST1 Left on the Sony Mixer |
| Tripple RCA Cable Yellow, Red & White | Falcon TV- Red Audio Cable | ST1 Right on the Sony Mixer |
| Tripple RCA Cable Yellow, Red & White | Falcon TV-Yellow AV Connector | Video Input A on the Sony Mixer |
| Single RCA Cable Yellow | Master Output on the Sony Mixer | AV INPUT on the Plasma's Receiver |
| Table Top Mic Stereo Mini Jack Connector | Table Top Microphone | MIC on the Falcon |
| Single Yellow RCA Cable | Elmo Document Camera | Camera 1 on the Falcon |
| 2 x Red & Black Speaker Cable | Speaker Left & Right on the Plasma | Speaker Output Terminals on the Sony Mixer |
| Plasma to Receiver Connector | Display out on the Plasma's Receiver | Display In on the Plasma |
| Presenter Microphone Cable | Microphone Receiver | Mic 1 on the Sony Mixer |
| Vocal Microphone Cable | Microphone Receiver | Mic 2 on the Sony Mixer |
| Power cables | Falcon Unit | Multiplug Socket |
| Power cables | Microphone Receiver | Multiplug Socket |
| Power cables | Microphone Receiver | Multiplug Socket |
| Power cables | Sony Mixer | Multiplug Socket |
| Power cables | Plasma's Receiver | Multiplug Socket |
| Power cables | Plasma | Multiplug Socket |
| Power cables | Elmo Document Camera | Multiplug Socket |

VCON Falcon with Sony Mixer

4. VCON Media Connect 9003

Media Connect 9003 Installation Document

1.  Remove the 9003 PC, Sony Camera and Voice Finder from the box.

2.  Install into the cabinet as per the Cabinet layout Diagram.

3.  Attach Voice Finders as per Instructions.

4.  Place Sony camera and Voice Finder on the shelf.

5.  Place the Voice Finder's Black box inside the cabinet as per the cabinet

layout Diagram.

6.  Place the batteries into the Wireless Keyboard and place the Infra red sensor

into the Cabinet as per the Cabinet layout Diagram.

7.  Install the Sony Mixer into the Cabinet as per the Cabinet Layout Document.

8.  Install the Plasma’s Receiver into the Cabinet as per the Cabinet Layout

Diagram.

9.  Mount the Plasma Screen as per the Room Layout Diagram.

10.Connect the system by using the Media Connect 9003 Set Up

Document.

Media Connect 9003 Set up Document

| Step | Installation Instruction | Connecting Cable Type/Name | From which Connection | To which Connection |
| --- | --- | --- | --- | --- |
| 1 | Connect | RJ45 cable connector | Voice Finder | Mic RJ45 Connector on Voice Finders Black Box |
| 2 | Connect | Double Joined Visca & DB9 Connector | S-Video on Camera | VID-TOP on MC 9003 |
| 3 | Connect | Double Joined Visca & DB9 Connector | Visca In on Camera | Com 1 on the MC 9003 |
| 4 | Connect | Single Yellow RCA Cable | Voice Finder's Black box- Speakers | VCR Audio IN on the MC 9003 |
| 5 | Connect | Double ended DB9 cable | Voice Finder's Black box- Data | Com 2 on the MC 9003 |
| 6 | Connect | RCA to Mini Stereo Jack Cable | Voice Finder's Black Box-Far Line In | Speaker on the MV 9003 |
| 7 | Connect | Single RCA to Double RCA Cable | Voice Finder's Black Box-Local Line Out | ST-1 on the Sony Mixer |
| 8 | Connect | Double ended male RGB Cnable | VGA Display on the MC 9003 | Plasma Monitor RGB IN |
| 9 | Connect | Double RCA Cable-Black & Yellow | Master Output on the Sony Mixer | Left & Right INPUTS on the Plasma's Receiver |
| 10 | Connect | Table Top Mic Stereo Mini Jack Connector | Table Top Microphone | MIC on the MC 9003 |
| 11 | Connect | Single Yellow RCA Cable | Elmo Document Camera | Camera on the MC 9003 |
| 12 | Connect | 2 x Red & Black Speaker Cable | Speaker Left & Right on the Plasma | Speaker Output Terminals on the Sony Mixer |
| 13 | Connect | Plasma to Receiver Connector | Display out on the Plasma's Receiver | Display In on the Plasma |
| 14 | Connect | Presenter Microphone Cable | Microphone Receiver | Mic 1 on the Sony Mixer |
| 15 | Connect | Vocal Microphone Cable | Microphone Receiver | Mic 2 on the Sony Mixer |
| 16 | Connect | Power cables | Camera | Multiplug Socket |
| 17 | Connect | Power cables | Voice Finder's Black Box | Multiplug Socket |
| 18 | Connect | Power cables | Media Connect 9003 | Multiplug Socket |
| 19 | Connect | Power cables | Sony Mixer | Multiplug Socket |
| 20 | Connect | Power cables | Plasma's Receiver | Multiplug Socket |
| 21 | Connect | Power cables | Plasma | Multiplug Socket |
| 22 | Connect | Power cables | Elmo Document Camera | Multiplug Socket |
| 23 | Connect | Power cables | Microphone Receiver | Multiplug Socket |
| 24 | Connect | Power cables | Microphone Receiver | Multiplug Socket |

5. VCON Media Connect 9003 with Sony Mixer